# FSM Irrigation Requirements

## 1. Asset Hierarchy

The hierarchy is easiest to read in tree form.

Property Account
└── System
    └── Point of Connection (Meta: Water Source)
        ├── Pump
        ├── Backflow
        ├── Master Valve
        ├── Flow Sensor
        └── Controller
            └── Zone : Valve : Station

### 1.1 Hierarchy Rules

| Rule | Requirement |
| --- | --- |
| Parent-child consistency | Every asset must use the correct parent type for its record type. |
| Property system cardinality | Each Property has exactly one System. |
| Point of Connection ownership | A Point of Connection must belong to exactly one System. |
| Pump ownership | A Pump must belong to exactly one Point of Connection. |
| Backflow ownership | A Backflow must belong to exactly one Point of Connection. |
| Master Valve ownership | A Master Valve must belong to exactly one Point of Connection. |
| Flow Sensor ownership | A Flow Sensor must belong to exactly one Point of Connection. |
| Controller ownership | A Controller must belong to exactly one Point of Connection. |
| Zone ownership | A Zone must belong to exactly one Controller. |
| System grouping | System is required and groups a property’s irrigation assets. |
| Naming | Zone display names should be normalized to Zone <number> to reduce user input. |
| Retire-only behavior | Assets should be retired, not hard deleted, when removed from active use. |
| Component modeling | Valve and station details are represented as zone-linked metadata; Point-of-Connection assets (Pump, Backflow, Master Valve, Flow Sensor) are explicit hierarchy children. |

## 2. Component Metadata

This section defines the prototype-driven metadata baseline for each irrigation asset type in the desktop setup flow (prototype/desktop/desktop_v3.1.html + property_record.js).

### 2.1 Common Metadata

| Field | Purpose | Applies To |
| --- | --- | --- |
| Name | Human-readable asset name shown in UI and hierarchy. | All assets (Zone name is system-generated) |
| Asset Type | Standard asset discriminator. | All assets |
| Status | Active / Retired lifecycle state. | All assets |
| Parent | Links the asset to the correct place in the hierarchy. | All assets except System root |
| Install Date | Optional lifecycle date captured in edit flow. | System, Point of Connection, Pump, Backflow, Master Valve, Flow Sensor, Controller, Zone |
| Description | Optional free-text context. | System, Point of Connection, Pump, Backflow, Master Valve, Flow Sensor, Controller, Zone |

### 2.2 System Metadata

| Metadata | Required | Notes |
| --- | --- | --- |
| Name | Yes | Root grouping label for the irrigation system. |
| Asset Type | Yes | Must be System. |
| Parent | No | Stored as root (parentId = null) and grouped under the selected Property context. |
| Description | No | Optional system context shown in detail panel. |
| Install Date | No | Useful for lifecycle context. |
| Serial Number | No | Displayed in detail panel; currently edit flow clears this value (known prototype gap). |
| Mainline Pipe Type | No | Descriptive pipe material (PVC, Poly, Copper, etc.). Optional; captured at setup. |
| Mainline Pipe Size | No | Descriptive pipe diameter (e.g., 1”, 1.5”). Optional; captured at setup. |

### 2.3 Controller Metadata

| Metadata | Required | Notes |
| --- | --- | --- |
| Name | Yes | Controller display name. |
| Asset Type | Yes | Must be Controller. |
| Parent | Yes | Parent is the Point of Connection. |
| Controller Label | Yes | Human-friendly identifier used by the tech. |
| Total Zones | Yes | Capacity / zone count for the controller. |
| Make / Model | No | Manufacturer and model information. |
| Connectivity Type | No | For example WiFi, Wired, or No Connectivity. |
| Smart Controller | No | Indicates whether the controller is smart / weather-based. |
| Controller App / Platform | No | Companion app name or platform name. |
| Install Date | No | Optional lifecycle date. |

### 2.4 Point of Connection Metadata

| Metadata | Required | Notes |
| --- | --- | --- |
| Name | Yes | Point-of-Connection display name. |
| Asset Type | Yes | Must be Point of Connection. |
| Parent | Yes | Parent is System. |
| Water Source Type | No | Meta field examples: Potable, Reclaimed, Well, Other. |
| Source Capacity | No | Optional capacity or pressure metadata for planning. |
| Install Date | No | Optional lifecycle date. |

### 2.5 Zone Metadata

| Metadata | Required | Notes |
| --- | --- | --- |
| Name | Yes | Automatically generated and normalized to Zone <number>. |
| Asset Type | Yes | Must be Zone. |
| Parent | Yes | Parent is the Controller. |
| Zone Number | Yes | Primary zone identifier; must be unique per Property. |
| Area Served | No | Human-readable area description. |
| Flow Rate (GPM) | No | Operational or design flow rate. |
| Primary Head Type | No | Useful when the zone is mapped to a head subtype. |
| Distribution Method | No | Spray / Rotor / Bubbler / Drip. Captured during inspection per Q6.3. |
| Lateral Pipe Type | No | Descriptive lateral/drip line material. Optional; examples: soaker hose, drip tape, mainline lateral. |
| Lateral Pipe Size | No | Descriptive diameter/designation. Optional. |
| Solenoid Resistance (Ω) | No | Last measured solenoid resistance in ohms. Nominal range 20–60 Ω; out-of-range flags valve diagnostic need. Written by inspection checkout via Q6.13c. |
| Install Date | No | Optional lifecycle date. |

### 2.6 Backflow Metadata

| Metadata | Required | Notes |
| --- | --- | --- |
| Name | Yes | Backflow display name. |
| Asset Type | Yes | Must be Backflow. |
| Parent | Yes | Parent is the Point of Connection. |
| Backflow Type | Yes | Examples: RPZ, DCV, PVB, Other. |
| Serial Number | No | Useful for compliance tracking. |
| Last Test Date | No | Compliance history. |
| Last Test Result | No | Options in prototype: Pass, Fail, Not Tested. |
| Next Test Due | No | Optional date for upcoming compliance activity. |
| Compliance Status | No | Current compliance state. |
| Testing Authority | No | Authority or vendor responsible for testing. |

### 2.7 Pump Metadata

| Metadata | Required | Notes |
| --- | --- | --- |
| Name | Yes | Pump display name. |
| Asset Type | Yes | Must be Pump. |
| Parent | Yes | Parent is the Point of Connection. |
| Operational Status | No | Running / Off / Faulted snapshot from inspection or service event. |
| Pressure (PSI) | No | Last observed pressure reading at inspection time. |
| Pump Type | No | Example values: Booster, Well, Transfer, Other. |
| Install Date | No | Optional lifecycle date. |

### 2.8 Master Valve Metadata

| Metadata | Required | Notes |
| --- | --- | --- |
| Name | Yes | Master valve display name. |
| Asset Type | Yes | Must be Master Valve. |
| Parent | Yes | Parent is the Point of Connection. |
| Operational Status | No | Open / Closed / Faulted snapshot from inspection or service event. |
| Valve Type | No | Example values: Normally Closed, Normally Open, Latching, Other. |
| Solenoid Resistance (Ω) | No | Measured value used for troubleshooting when applicable. |
| Install Date | No | Optional lifecycle date. |

### 2.9 Flow Sensor Metadata

| Metadata | Required | Notes |
| --- | --- | --- |
| Name | Yes | Flow sensor display name. |
| Asset Type | Yes | Must be Flow Sensor. |
| Parent | Yes | Parent is the Point of Connection. |
| Functional Status | No | Connected / Not Connected / Faulted. |
| Sensor Model | No | Manufacturer/model value used for troubleshooting. |
| Flow Reading (GPM) | No | Last observed live reading when available. |
| Install Date | No | Optional lifecycle date. |

### 2.10 Zone Subcomponent Metadata (Valve + Station)

| Metadata | Required | Notes |
| --- | --- | --- |
| Valve Type | No | Zone valve hardware classification. |
| Valve Size | No | Physical size/designation. |
| Valve Location Notes | No | Field notes for locating the valve box and valve. |
| Station Identifier | No | Controller station/channel mapped to this zone. |
| Station Wire Path Notes | No | Wiring trace notes for troubleshooting. |
| Station Electrical Status | No | Good / Open / Short / Intermittent. |

### 2.11 Inspection-Linked Metadata

These fields come from the Standard question library and should live alongside the asset metadata model so setup, inspection, and reporting all refer to the same record shape.

| Asset Type | Metadata | Required | Notes |
| --- | --- | --- | --- |
| System | Mainline Pipe Type | No | Descriptive pipe material. Captured during setup and referenced in inspection reporting. |
| System | Mainline Pipe Size | No | Descriptive diameter/designation. Captured during setup and referenced in inspection reporting. |
| Point of Connection | Water Source Type | Yes | Potable / Reclaimed / Well / Pond / Other. Captured by inspection Q3.1 and stored on the Point of Connection asset. |
| Point of Connection | Source Capacity | No | Optional capacity or pressure metadata for planning. |
| Pump | Operational Status | No | Captured when pump equipment is present and inspected. |
| Pump | Pressure (PSI) | No | Captured from field reading during inspection/service. |
| Backflow | Backflow Type | Yes | RPZ / DCV / PVB / Other. Captured by inspection Q4.1 and stored on the Backflow asset. |
| Master Valve | Operational Status | No | Captured when master valve is installed at the site. |
| Master Valve | Solenoid Resistance (Ω) | No | Diagnostic measurement captured when troubleshooting is performed. |
| Flow Sensor | Functional Status | No | Connected / Not Connected / Faulted state captured during inspection. |
| Flow Sensor | Flow Reading (GPM) | No | Current reading captured when sensor telemetry is available. |
| Controller | Controller Label | Yes | Human-friendly controller identifier used in the field. Captured by inspection Q5.2. |
| Controller | Total Zones | Yes | Capacity / zone count for the controller. Captured by inspection Q5.3. |
| Controller | Make / Model | No | Manufacturer and model information. Captured by inspection Q5.1 when discovered or corrected. |
| Controller | Smart Controller | No | Indicates whether the controller is smart / weather-based. Captured by inspection Q5.8. |
| Controller | Flow Sensor Connected and Functional | No | Controller equipment-package metadata. Captured by inspection Q5.9 when applicable. |
| Zone | Zone Number | Yes | Primary zone identifier; must be unique per Property. Captured by inspection Q6.1. |
| Zone | Area Served | No | Human-readable area description. Captured by inspection Q6.2. |
| Zone | Distribution Method | No | Spray / Rotor / Bubbler / Drip. Captured by inspection Q6.3. |
| Zone | Landscape Type | No | Turf / Bed / Color or similar setup label used by the field team. Captured by inspection Q6.4. |
| Zone | # of Heads | No | Zone component metadata. Captured by inspection Q6.5. |
| Zone | Solenoid Resistance (Ω) | No | Last measured solenoid resistance in ohms. Captured by inspection Q6.13c. |
| Zone | Station Identifier | No | Controller station/channel mapping for this zone. |
| Zone | Station Electrical Status | No | Wiring health result captured during troubleshooting. |

### 2.12 Inspection Checklist Items

These are visit-scoped observations or actionable finding prompts. They should not be treated as static asset metadata, even when they are recorded against an asset-scoped inspection response.

| Asset Type | Checklist Item | Required | Notes |
| --- | --- | --- | --- |
| System | Visible mainline leak? | Yes | Mainline/distribution summary. |
| System | Mainline pressure stable? | No | System-level condition check. |
| System | Isolation valves operational? | No | System-level condition check. |
| System | Quick coupler valves operational (where applicable)? | No | System-level condition check. |
| Point of Connection | Water restrictions in place? | Yes | Compliance check captured during the visit. |
| Point of Connection | Restriction details | Conditional | Required when restrictions are in place. |
| Pump | Pump operational? | Conditional | Required when a Pump component exists under the Point of Connection. |
| Pump | Pump pressure (PSI) | No | Diagnostic reading taken during the visit. |
| Pump | Pump cycling/noise abnormal? | No | Flags short-cycling or abnormal operating behavior. |
| Backflow | Visible damage? | Yes | Visual condition check. |
| Backflow | Leaks at backflow assembly? | Yes | Visual condition check. |
| Backflow | Test required this visit? | Yes | Drives test-specific branching. |
| Backflow | Test passed? | Conditional | Required when testing is performed. |
| Backflow | Test certificate uploaded? | Conditional | Required when testing is performed. |
| Backflow | Insulation / freeze protection in place? | No | Seasonal checklist prompt. |
| Master Valve | Master valve operational? | Conditional | Required when a Master Valve component exists. |
| Master Valve | Master valve leaking? | No | Condition check for seepage or failure. |
| Master Valve | Master valve manual override functional? | No | Confirms serviceability during diagnostics. |
| Flow Sensor | Flow sensor installed and connected? | Conditional | Required when a Flow Sensor component exists. |
| Flow Sensor | Flow sensor reading plausible? | No | Confirms observed reading matches expected zone/system behavior. |
| Flow Sensor | Flow sensor fault/alarm present? | No | Captures diagnostic state for follow-up. |
| Controller | Controller power state | Yes | On / Off. |
| Controller | Backflow state | Yes | On / Off. |
| Controller | Meter state | Yes | On / Off. |
| Controller | Rain / freeze sensor working? | Yes | Controller accessory check. |
| Controller | Programs match contract / season? | Yes | Programming audit. |
| Controller | Adjustments made this visit? | Yes | Drives adjustment notes. |
| Controller | Adjustment notes | Conditional | Required when adjustments were made. |
| Controller | System winterized / drained? | Conditional | Winterization-only branch. |
| Zone | Minutes / zone (runtime) | No | Operational/programming observation. |
| Zone | No issues found | Yes | Mutually exclusive with failure items below. |
| Zone | Broken head | Yes | Failure item; creates an actionable checklist finding (can map to WOLI in AM flow). |
| Zone | Broken / clogged nozzle | Yes | Failure item; creates an actionable checklist finding (can map to WOLI in AM flow). |
| Zone | Sunken / tilted head | Yes | Failure item; creates an actionable checklist finding (can map to WOLI in AM flow). |
| Zone | Head not retracting | Yes | Failure item; creates an actionable checklist finding (can map to WOLI in AM flow). |
| Zone | Head not rotating | Yes | Failure item; creates an actionable checklist finding (can map to WOLI in AM flow). |
| Zone | Overwatering onto hardscape | No | Region-specific delta. |
| Zone | Lateral leak | Yes | Failure item; creates an actionable checklist finding (can map to WOLI in AM flow). |
| Zone | Valve not activating | Yes | Failure item; creates an actionable checklist finding (can map to WOLI in AM flow). |
| Zone | Seeping valve | Yes | Failure item; creates an actionable checklist finding (can map to WOLI in AM flow). |
| Zone | Bad solenoid | Yes | Failure item; creates an actionable checklist finding (can map to WOLI in AM flow). |
| Zone | Valve box lid missing | Yes | Failure item; creates an actionable checklist finding (can map to WOLI in AM flow). |
| Zone | Broken drip line | Conditional | Drip-only branch. |
| Zone | Drip emitters / filter / regulator OK? | Conditional | Drip-only branch. |
| Zone | Station wiring fault observed? | No | Captures station-level electrical issues linked to the zone. |
| Zone | Repairs made on site this visit? | Yes | Captures whether the tech completed corrective work. |
| Zone | Repair description | Conditional | Required when repairs were made. |
| Zone | Zone notes | No | Free-form inspection notes. |

### 2.13 Component Metadata (Non-Hierarchy)

Valve, head, drip, station, and other zone-level equipment details are modeled as metadata attached to Standard hierarchy assets. This keeps the hierarchy fixed at System -> Point of Connection -> Controller -> Zone (with Pump, Backflow, Master Valve, and Flow Sensor as Point-of-Connection children) while still supporting detailed operational capture.

Examples: 1. Zone-level metadata: valve type, head mix, emitter profile, lateral pipe attributes. 2. Controller-level metadata: sensor package, flow monitoring setup. 3. Backflow-level metadata: compliance and testing attributes.